UMAT-OTI JSON Configuration Guide

# 1. Motivation and Program Purpose

Complex UMATs often require Jacobians such as DDSDDE. Deriving and coding these derivatives by hand can be slow, difficult, and error-prone. The purpose of this program is to help transform an existing UMAT so that the required derivatives can be obtained automatically using OTI-based differentiation.

The user provides a JSON file that describes the UMAT source file, the derivative that must be computed, the variables that must become OTI variables, and the lines of the original analytical derivative block that must be replaced. The program reads this JSON contract and produces a transformed UMAT source file plus reports describing what was changed.

# 2. Code the User Has to Run

The simplest way to run one user-defined case is to use the wrapper script from the root of the package folder:

/usr/bin/python3.12 transform_from_json.py user_jsons/my_umat.json --out umat_oti_workspace/my_umat

In this command:

| Part | Meaning |
| --- | --- |
| /usr/bin/python3.12 | Python interpreter used to run the script. On another machine, this may simply be python or python3. |
| transform_from_json.py | Script that reads one compact JSON file and launches the UMAT transformation. |
| user_jsons/my_umat.json | Path to the JSON file written by the user. |
| --out umat_oti_workspace/my_umat | Output folder where the transformed UMAT and reports will be written. |

For multiple JSON files, the package also provides run_json_pipeline.py. This is useful when several cases should be transformed in one run. The user edits the settings at the top of that script:

JSON_INPUT_PATH = ROOT / "user_jsons"
OUTPUT_DIRECTORY = ROOT / "umat_oti_workspace" / "my_run"
RUN_VALIDATION = False

Then the user runs:

/usr/bin/python3.12 run_json_pipeline.py

# 3. Program Inputs

The program needs the following inputs:

| Input | What it is | Where it is provided |
| --- | --- | --- |
| UMAT source file | Original Fortran UMAT to transform. | In the JSON field source. |
| JSON configuration file | Contract that tells the program how to transform the UMAT. | Given as the command argument, for example user_jsons/my_umat.json. |
| Output directory | Folder where the transformed files and reports are written. | Given with --out, or inside run_json_pipeline.py. |
| Validation setting | Optional setting that decides whether Abaqus validation is also run. | RUN_VALIDATION inside run_json_pipeline.py. |

# 4. How the JSON File Must Be Built

The JSON file is a structured description of the transformation. It should be written by copying a template and replacing the placeholder values with information from the user’s own UMAT.

{
  "name": "my_new_umat",
  "source": "/absolute/path/to/your_umat.for",
  "jacobian": {
    "seed": "DSTRAN",
    "output": "STRESS",
    "target": "DDSDDE"
  },
  "description": "Short description of this UMAT case.",
  "promote": ["STRESS"],
  "constant": ["PROPS", "STATEV"],
  "real": ["DDSDDE", "NTENS"],
  "replace": ["START-END"],
  "ntens": 6,
  "order": 1
}

## 4.1 Required Fields

| Field | Purpose | How to choose it |
| --- | --- | --- |
| name | Short case name used in reports and output folders. | Use a clear label, such as elastic, UMAT_HIN, or my_crystal_plasticity_umat. |
| source | Path to the original Fortran UMAT file. | Use an absolute path when possible. A relative path can be used if it is correct relative to the JSON file location. |
| jacobian | Defines the derivative being computed. | For standard UMAT tangent stiffness, use seed = DSTRAN, output = STRESS, target = DDSDDE. |
| promote | Variables that must become OTI variables so derivatives can propagate through the calculations. | Include stress-update variables and intermediate variables that depend on the seeded input. |
| constant | Variables that are used in the calculation but are not differentiated. | Include material properties and fixed inputs, such as PROPS, TEMP, TIME, STRAN, and constants. |
| real | Variables that must remain normal real variables. | Include output storage arrays and indices, especially DDSDDE and NTENS. |
| replace | Line range of the old analytical derivative block that will be replaced. | Use 1-based inclusive line numbers, for example "83-87". |
| ntens | Number of stress/strain components to seed. | Usually 6 for 3D and 4 for plane strain/axisymmetric-style examples in this package. |
| order | OTI differentiation order. | Use 1 for the current first-order Jacobian workflow. |

## 4.2 The jacobian Block

The jacobian block tells the program which derivative is needed. It has three entries:

"jacobian": {
  "seed": "DSTRAN",
  "output": "STRESS",
  "target": "DDSDDE"
}

| Entry | Meaning |
| --- | --- |
| seed | Independent variable. This is the variable perturbed with OTI directions. For DDSDDE, this is normally DSTRAN. |
| output | Dependent variable. This is the variable whose derivative is extracted. For DDSDDE, this is normally STRESS. |
| target | Real UMAT variable that receives the extracted derivative. For the standard tangent, this is DDSDDE. |

In simple terms, this block means: compute how STRESS changes with respect to DSTRAN, then place that derivative into DDSDDE.

## 4.3 The Variable Role Lists

The most important part of the JSON is assigning variables to the correct role list. Each variable should appear in only one of these lists: promote, constant, or real.

| List | Put variables here when... | Typical examples |
| --- | --- | --- |
| promote | The variable participates in the calculation and must carry derivative information. | STRESS, DSTRESS, trial stresses, residual quantities, plastic strain increments, local intermediate arrays. |
| constant | The variable is used by the calculation but should not be differentiated. | PROPS, TEMP, TIME, STRAN, PREDEF, material constants, fixed input arrays. |
| real | The variable should remain real because it is an index, counter, metadata value, or final storage for extracted derivatives. | DDSDDE, NTENS, NDI, NSHR, NOEL, NPT, KSTEP, KINC, loop counters I, J, K. |

## 4.4 How to Choose promote, constant, and real

Start from the line where DSTRAN is used and follow the calculations until STRESS is updated.

Add variables that are computed from DSTRAN and later affect STRESS to promote.

Add material parameters and fixed Abaqus inputs to constant.

Add integer counters, dimensions, names, and bookkeeping variables to real.

Add DDSDDE to real because DDSDDE is the target where derivatives are stored after extraction.

Do not repeat the same variable in more than one list.

Example of a small but realistic role assignment:

"promote": ["STRESS", "DSTRESS", "TRVAL"],
"constant": ["PROPS", "STATEV", "STRAN", "TEMP", "TIME"],
"real": ["DDSDDE", "NTENS", "NDI", "NSHR", "I", "J", "K"]

## 4.5 The replace Field

The replace field tells the program which lines of the original UMAT contain the old analytical derivative assignment. Those lines are removed and replaced by derivative extraction code generated from the OTI evaluation.

"replace": ["83-87"]

The range is inclusive and uses 1-based line numbers. Therefore, "83-87" means line 83, 84, 85, 86, and 87 of the original source file. This range should normally cover only the old DDSDDE assignment block, not the entire stress update.

| Good replace range | Bad replace range |
| --- | --- |
| Only the old DDSDDE calculation block. | The entire UMAT subroutine. |
| Only lines that assign or build the analytical tangent. | Lines that update STRESS or STATEV. |
| A small clear range, such as "273-288". | A broad unclear range that removes required computations. |

## 4.6 ntens and order

| Field | Meaning | Common value |
| --- | --- | --- |
| ntens | Number of stress/strain components used by the UMAT. It controls the number of DSTRAN directions seeded. | 6 for full 3D; 4 for the smaller plane-style examples used in the package. |
| order | Derivative order for OTI calculations. | 1 for first-order Jacobians. |

## 4.7 Optional Fields

Most users should start with the required fields only. Optional fields can be added after the base JSON works.

| Optional field | When to use it |
| --- | --- |
| description | Add a short explanation of the case. This is useful for reports. |
| umat | Use only if the UMAT entry routine name is not detected correctly. |
| validation | Use to define what should be compared during validation and what tolerance should be used. |
| constitutive_jacobians | Use only for advanced cases where additional helper-level constitutive derivatives must be extracted. |
| helper_surfaces | Use only when helper routine outputs must be mapped back to caller-level variables. |

"validation": {
  "compare": ["STRESS", "STATEV", "CONVERGENCE"],
  "absolute_tolerance": 0.0001
}

## 4.8 Complete Minimal Example

This is a complete minimal JSON example. A real UMAT may need longer promote, constant, and real lists, but the structure stays the same.

{
  "name": "elastic_test",
  "source": "../UMATs/UMATs/ICP/elasticity/elastic.f",
  "jacobian": {
    "seed": "DSTRAN",
    "output": "STRESS",
    "target": "DDSDDE"
  },
  "description": "Elastic UMAT transformation test.",
  "promote": [
    "DSTRESS",
    "STRESS",
    "TRVAL"
  ],
  "constant": [
    "PROPS",
    "STATEV",
    "STRAN",
    "TEMP",
    "TIME"
  ],
  "real": [
    "DDSDDE",
    "NTENS",
    "NDI",
    "NSHR",
    "I",
    "J",
    "K"
  ],
  "replace": ["83-87"],
  "ntens": 4,
  "order": 1
}

## 4.9 Recommended JSON Writing Procedure

Copy templates/new_umat_minimal_template.json into user_jsons/ and rename it for the UMAT case.

Set name to a clear case name.

Set source to the path of the original Fortran UMAT file.

Keep jacobian as DSTRAN to STRESS into DDSDDE for a standard UMAT tangent.

Find the original DDSDDE analytical assignment block and write its line range in replace.

Set ntens to the number of stress/strain components used by the case.

Build promote, constant, and real by following the stress-update calculation.

Check that the JSON syntax is valid and that no variable appears in more than one role list.

Run transform_from_json.py and inspect transform_report.json if the transformation fails.

# 5. Program Outputs

After a successful transformation, the output directory contains the transformed UMAT source and reports. The exact names can vary by case, but the main outputs are:

| Output | Meaning |
| --- | --- |
| *_oti.f or transformed UMAT source | Fortran UMAT source generated from the original UMAT and the JSON contract. |
| transform_report.json | Main report describing the transformation, selected variables, generated files, and any warnings. |
| OTI support modules | Fortran support code needed by the transformed UMAT. |
| compile hints or generated build information | Information that helps compile the transformed UMAT. |
| pipeline_status.json | Summary file written by run_json_pipeline.py when multiple configs are processed. |
| validation_report.json | Only produced when validation is enabled. |
| comparison_report.json | Only produced when validation is enabled and comparison is completed. |

If RUN_VALIDATION = False, the expected outputs are the transformed source and transformation reports. Validation reports are expected only when validation is enabled.